Lab Title: Study of different types of Network Cables and practical implementation of Cross-wired cable and Straight-through cable using clamping tool.

Objective:

To study different types of network cables and practically implement the making of Straight-through and Cross-over cables using a crimping (clamping) tool.

Apparatus/Software Required:

- Twisted Pair Cable (Category 5e or Category 6 – UTP)

- RJ-45 Connectors

- Crimping Tool (Clamping Tool)

- Cable Tester (for verification)

- Cutter/Stripper

Theory:

In computer networks, cables are the physical medium through which data signals travel. Commonly used network cables are:

- Coaxial Cable – An older technology with a central conductor, insulating layer, metallic shield, and outer covering. Mostly obsolete today.

- Twisted Pair Cable (UTP/STP) – Consists of pairs of wires twisted together to reduce electromagnetic interference. Types include Cat5, Cat5e, Cat6.

- Fiber Optic Cable – Uses light pulses instead of electrical signals, supports very high bandwidth and long-distance communication.

Among these, UTP (Unshielded Twisted Pair) cables are most widely used in LANs.

- Straight-through Cable:

Connects devices of different types (e.g., PC to Switch, PC to Router). Pin configuration is same at both ends, usually following TIA/EIA-568B standard.

- Cross-over Cable:

Connects devices of same type (e.g., PC to PC, Switch to Switch). In this, transmit and receive wires are swapped at one end (568A on one side and 568B on the other).

Standards:

- 568A Pinout: Green pair first

- 568B Pinout: Orange pair first

Network Cable Pin Configuration

| Pin Number | 568A Wire Color | 568B Wire Color |
| --- | --- | --- |
| 1 | White/Green | White/Orange |
| 2 | Green | Orange |
| 3 | White/Orange | White/Green |
| 4 | Blue | Blue |
| 5 | White/Blue | White/Blue |
| 6 | Orange | Green |
| 7 | White/Brown | White/Brown |
| 8 | Brown | Brown |

Procedure:

Step 1: Cable Preparation

- Cut required length of UTP cable.

- Strip about 1 inch of the outer insulation using cable stripper.

- Untwist the wire pairs and arrange them according to pinout standard (568A or 568B).

Step 2: Connector Insertion

- Align the wires in correct order and flatten them.

- Insert the wires into the RJ-45 connector carefully.

Step 3: Crimping

- Place the RJ-45 connector into the crimping tool.

- Press firmly to lock the wires into the connector.

Step 4: Testing

- Repeat process for both ends.

- For Straight-through Cable: use 568B on both ends.

- For Cross-over Cable: use 568A on one end and 568B on the other.

- Verify using a cable tester to ensure connectivity.

Observation:

- Straight-through cable successfully tested between PC and switch/router.

- Cross-over cable successfully tested between two PCs directly.

- Cable tester showed correct continuity for all 8 wires.

Result:

Straight-through and cross-over cables were successfully made using UTP cable, RJ-45 connectors, and a crimping tool. The cables were verified using a cable tester for proper connectivity.

Conclusion:
This lab provided practical knowledge of cabling standards, highlighting the importance of correct cables for efficient LAN setup.