Lab Title: Simulate a network with three routers and configure RIP (Routing Information Protocol). Verify routing tables and connectivity.

Objective

To simulate a three-router network, configure RIP for dynamic routing, and verify routing tables and connectivity across all LANs.

Apparatus/software required:

- Cisco Packet Tracer

- 6 PCs

- 3 PT-Switches

- 3 PT-Routers

Network Topology Diagram

Fig: Three-Router RIP Network Topology

IP Addressing Scheme

For PCs

| S.NO | Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| 1 | PC0 | 192.168.27.2 | 255.255.255.0 | 192.168.27.1 |
| 2 | PC1 | 192.168.27.3 | 255.255.255.0 | 192.168.27.1 |
| 3 | PC2 | 192.168.28.2 | 255.255.255.0 | 192.168.28.1 |
| 4 | PC3 | 192.168.28.3 | 255.255.255.0 | 192.168.28.1 |
| 5 | PC4 | 192.168.29.2 | 255.255.255.0 | 192.168.29.1 |
| 6 | PC5 | 192.168.29.3 | 255.255.255.0 | 192.168.29.1 |

For Routers

| S.NO | Router | Interface | IPv4 Address | Subnet Mask |
| --- | --- | --- | --- | --- |
| 1 | Router0 | FastEthernet0/0 | 192.168.27.1 | 255.255.255.0 |
|  |  | Serial2/0 | 10.0.0.1 | 255.0.0.0 |
| 2 | Router1 | FastEthernet0/0 | 192.168.28.1 | 255.255.255.0 |
|  |  | Serial2/0 | 10.0.0.2 | 255.0.0.0 |
|  |  | Serial2/1 | 11.0.0.1 | 255.0.0.0 |
| 3 | Router2 | FastEthernet0/0 | 192.168.29.1 | 255.255.255.0 |
|  |  | Serial2/0 | 11.0.0.2 | 255.0.0.0 |

Procedure

Step 1: Device Placement and Cabling

- Place 6 PCs, 3 switches, 3 routers.

- PCs → Switch → Router.

- Connect R0–R1 and R1–R2 via serial links.

- Use auto cabling.

Step 2: Assign IP Addresses on PCs

Enter IP, subnet mask, and default gateway as per the table above, for ex:

Step 3: Configure Router Interfaces

- On each Router → Config → FastEthernet0/0 → assign IP.

- On Serial ports, assign IPs and turn ports ON.

Step 4: Configure RIP on Routers

- On Router0:

Router> enable

Router# configure terminal

Router(config)# router rip

Router(config-router)# version 2

Router(config-router)# network 10.0.0.0

Router(config-router)# network 192.168.27.0

- On Router1:

Router> enable

Router# configure terminal

Router(config)# router rip

Router(config-router)# version 2

Router(config-router)# network 10.0.0.0

Router(config-router)# network 10.0.1.0

Router(config-router)# network 192.168.28.0

- On Router2:

Router> enable

Router# configure terminal

Router(config)# router rip

Router(config-router)# version 2

Router(config-router)# network 10.0.1.0

Router(config-router)# network 192.168.29.0

Step 5: Verify Routing Tables and Test Connectivity

PC2>Ping 192.168.27.2

PC5>Ping 192.168.28.2

Observation

RIP configured on all routers; routing tables updated; ping successful.

Result

Dynamic routing enabled full LAN-to-LAN communication without static routes.

Conclusion

RIP efficiently updated routes across three routers, ensuring seamless connectivity.