Lab Title: Configure a Basic Firewall in Cisco Packet Tracer to Control and Filter Traffic Between Two Networks

Objective:
Configure a firewall on a server in Cisco Packet Tracer to block ping requests while allowing web traffic, demonstrating basic traffic control.

Apparatus / Software Required

- 1 PT-Server (Server0)

- 1 PT-Switch

- 3 PCs (PC0, PC1, PC2)

- Cisco Packet Tracer software

Network Topology Diagram

IP Addressing Table:

| S.NO | Device | IPv4 Address | Subnet Mask |
| --- | --- | --- | --- |
| 1 | Server | 192.168.27.1 | 255.255.255.0 |
| 2 | PC0 | 192.168.27.2 | 255.255.255.0 |
| 3 | PC1 | 192.168.27.3 | 255.255.255.0 |
| 4 | PC2 | 192.168.27.4 | 255.255.255.0 |

Procedure

Step 1: Build connection

- Place 1 switch, 3 PC’s and 1 Server

- Connect PC’s to switch and Server to switch

Step 2: Configure IP Addresses

Configure each PC and the server with IPv4 addresses according to the table.

For example, PC0:

Step 3: Configure Firewall on Server

- Click on Server0 → Desktop → Firewall (IPv4)

- Turn Firewall Services ON

- Configure the following rules:

- Deny ‘ICMP’ protocol

  - Remote IP: 0.0.0.0

  - Remote Wildcard Mask: 255.255.255.255

- Allow ‘IP’ protocol

  - Remote IP: 0.0.0.0

  - Remote Wildcard Mask: 255.255.255.255

- Add the rules in sequence (first Deny ICMP, then Allow IP).

Step 4: Verify the Network

Test with Ping Command:

- Go to PC2 → Desktop → Command Prompt

- PC2> ping 192.168.27.1

Test with Web Browser:

- On PC2 → Desktop → Web Browser

- Enter 192.168.27.1 in the URL bar.

Observations

- Pings from PCs to the Server are blocked by the firewall.

- Web browsing works successfully despite ICMP being blocked.

- This demonstrates selective traffic filtering using firewall rules.

Result

A basic firewall was successfully configured in Cisco Packet Tracer to block ICMP traffic while allowing HTTP access.

Conclusion:
The lab showed that a firewall can control network traffic by blocking pings and allowing web browsing, highlighting its role in network security.