Lab Title: Design and configure a small LAN using Packet Tracer with at least 4 PCs, a switch, and a router. Assign IP addresses manually and test connectivity using ping.

Objective

To design and configure a small Local Area Network (LAN) consisting of 4 PCs, a switch, and a router. Assign IP addresses manually and test the connectivity using the ping command.

Apparatus / Software Required

• Cisco Packet Tracer
• 4 PCs
• 1 Switch
• 1 Router
• Copper Straight-Through Cables

Network Topology Diagram:

(Place 4 PCs connected to the Switch; Switch connected to Router FastEthernet0/0)

IP Addressing Scheme:

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC0 | Fa0 | 192.127.7.10 | 255.255.255.0 | 192.127.7.1 |
| PC1 | Fa0 | 192.127.7.11 | 255.255.255.0 | 192.127.7.1 |
| PC2 | Fa0 | 192.127.7.12 | 255.255.255.0 | 192.127.7.1 |
| PC3 | Fa0 | 192.127.7.13 | 255.255.255.0 | 192.127.7.1 |
| Router | Fa0/0 | 192.127.7.1 | 255.255.255.0 | — |

Where, Fa0: FastEthernet0

Procedure

Step 1: Device Placement and Cabling

• Place 4 PCs, a Switch, and a Router in the Packet Tracer workspace.
• Use Copper Straight-Through Cables to connect:

- PC’s  to Switch

- Switch to Router Fa0/0

Step 2: IP Address Assignment on PCs

• Click on each PC → Desktop → IP Configuration.

• Enter the IP address, subnet mask, and default gateway as per the table above.

Step 3: Configure Router Interface

• Click the Router → Config → FastEthernet0/0

Step 4: Test Connectivity

• Open Command Prompt on each PC

• Use the ping command to test connectivity:

Observation

Each PC successfully sends and receives ping responses to/from:
• The Router (192.127.7.1)
• Other PCs on the same network (192.127.7.10 - 13)

Result

A small LAN with 4 PCs, a switch, and a router was successfully designed and configured. Manual IP addressing (all within 192.127.7.0/24) was applied, and connectivity was verified using the ping command.

Conclusion:
This lab demonstrated setting up a basic LAN with static IPs and testing communication, forming a foundation for more complex network topologies.