Lab Title: To simulate and configure different network topologies (Star, Ring, Bus, Mesh, and Hybrid) using Cisco Packet Tracer.

Objectives

- To understand the design and working of basic network topologies.

- To configure IP addresses for devices using 192.127.7.0/24.

- To test connectivity with the ping command.

Apparatus / Software Required

- Cisco Packet Tracer

- PCs (minimum 4–6 for each topology)

- Switches, Hubs, Routers (as required)

- Copper Straight-Through and Cross-Over Cables

(a) Star Topology

In a Star topology, all devices are connected to a central device (switch/hub).

Devices Used

- 1 Switch

- 4 PCs

Network topology Diagram

Fig: Star Topology

IP Addressing Scheme

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC0 | Fa0 | 192.127.7.10 | 255.255.255.0 | 192.127.7.1 |
| PC1 | Fa0 | 192.127.7.11 | 255.255.255.0 | 192.127.7.1 |
| PC2 | Fa0 | 192.127.7.12 | 255.255.255.0 | 192.127.7.1 |
| PC3 | Fa0 | 192.127.7.13 | 255.255.255.0 | 192.127.7.1 |
| Router | Fa0/0 | 192.127.7.1 | 255.255.255.0 | — |

Procedure

Step 1: Connect four PCs to a switch with straight-through cables, and link the switch to a router

Step 2: Configure IP addresses as per the table.

Step 3: Test Connectivity

(b) Ring Topology

In this Ring topology, 4 switches are connected in a closed loop (each switch connected to two other switches). Each switch has 1 PC connected to it, so the PCs communicate through the ring of switches.

Devices Used

- 4 Switches

- 4 PCs

- Copper Straight-Through cables (PC → Switch)

- Copper Cross-Over cables (Switch → Switch in ring)

Network Topology Diagram

Fig: Ring Topology

IP Addressing Scheme

| Device | Connected Switch | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC0 | Switch0 | 192.127.7.20 | 255.255.255.0 | 192.127.7.1 |
| PC1 | Switch1 | 192.127.7.21 | 255.255.255.0 | 192.127.7.1 |
| PC2 | Switch2 | 192.127.7.22 | 255.255.255.0 | 192.127.7.1 |
| PC3 | Switch3 | 192.127.7.23 | 255.255.255.0 | 192.127.7.1 |

Procedure

Step1: Place Devices

  - Place 4 PCs and 4 Switches in Packet Tracer.

  - Connect each PC to its switch using a straight-through cable.

  - Connect the switches in a ring:

    - Switch0 ↔ Switch1

    - Switch1 ↔ Switch2

    - Switch2 ↔ Switch3

    - Switch3 ↔ Switch0

Step 2: Assign IP Addresses

  - On each PC → Desktop → IP Configuration → enter the values from the table.

Step 3: Test Connectivity

(c) Bus Topology

In this Bus topology, 5 switches are connected in a line (simulating a single shared communication medium). Each switch has 1 PC connected to it.

Devices Used

- 5 Switches

- 5 PCs

- Copper Straight-Through cables (PC → Switch)

- Copper Cross-Over cables (Switch → Switch in line)

Network Topology Diagram

Fig: Bus Topology

IP Addressing Scheme

| Device | Connected Switch | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC0 | Switch0 | 192.127.7.30 | 255.255.255.0 | 192.127.7.1 |
| PC1 | Switch1 | 192.127.7.31 | 255.255.255.0 | 192.127.7.1 |
| PC2 | Switch2 | 192.127.7.32 | 255.255.255.0 | 192.127.7.1 |
| PC3 | Switch3 | 192.127.7.33 | 255.255.255.0 | 192.127.7.1 |
| PC4 | Switch4 | 192.127.7.34 | 255.255.255.0 | 192.127.7.1 |

Procedure

Step 1: Place 5 switches and 5 PCs, connect PCs to switches with straight-through cables, and connect switches in a line with cross-over cables.

Step 2: Assign IP addresses to all PCs as per the table.

Step 3: Test connectivity

Observation

- All PCs could communicate successfully along the bus.

- STP prevented any accidental loops when switches were connected in line.

(d) Mesh Topology

In this Mesh topology, 4 switches are fully interconnected with every other switch, and each switch has 1 PC connected to it. This ensures maximum redundancy.

Devices Used

- 4 Switches

- 4 PCs

- Copper Straight-Through cables (PC → Switch)

- Copper Cross-Over cables (Switch → Switch fully connected)

Network Topology Diagram

Fig: Mesh Topology

IP Addressing Scheme

| Device | Connected Switch | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC0 | Switch0 | 192.127.7.40 | 255.255.255.0 | 192.127.7.1 |
| PC1 | Switch1 | 192.127.7.41 | 255.255.255.0 | 192.127.7.1 |
| PC2 | Switch2 | 192.127.7.42 | 255.255.255.0 | 192.127.7.1 |
| PC3 | Switch3 | 192.127.7.43 | 255.255.255.0 | 192.127.7.1 |

Procedure

Step 1: Place 4 switches and 4 PCs, connect PCs to switches with straight-through cables, and interconnect switches with cross-over cables.

Step 2: Assign IP addresses as per the table.

Step 3: Test connectivity

(e) Hybrid Topology

The bus section connects to the ring section, forming a Hybrid topology.

Devices Used

- Bus section: 5 Switches + 5 PCs

- Ring section: 4 Switches + 4 PCs

- Copper Straight-Through cables (PC → Switch)

- Copper Cross-Over cables (Switch → Switch)

Network Topology Diagram

Fig: Hybrid Topology

IP Addressing Scheme

| Device | Connected Switch | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Bus PCs | Switch0–4 | 192.127.7.30–34 | 255.255.255.0 | 192.127.7.1 |
| Ring PCs | Switch5–8 | 192.127.7.20–23 | 255.255.255.0 | 192.127.7.1 |

Procedure

Step 1:

Build the bus in line, the ring in a closed loop, and connect one switch from each section with a cross-over cable.

Step 2: Assign IP addresses as per the table.

Step 3: Test connectivity:

  - Ping from Bus PCs → Ring PCs

  - Ping within Bus and Ring sections

Observations:

- Star: PCs communicate via a central switch.

- Ring: Packets travel through each PC in sequence.

- Bus: PCs share a common hub.

- Mesh: PCs have direct links with all others.

- Hybrid: Combines star and bus sections.

Result:
Various network topologies were successfully simulated and tested in Packet Tracer.

Conclusion:
This lab highlighted the differences between topologies, their configurations, and connectivity methods, providing insight for designing efficient, scalable networks