Lab Title: Bus and Star Topology Connection

Objective

To design and configure a network in Cisco Packet Tracer that connects a Bus topology and a Star topology using a router. IP addresses are manually assigned using CLI mode, and connectivity is verified between the two networks using ping.

Apparatus/Software Required

- Cisco Packet Tracer

- 5 PCs (2 in Bus, 3 in Star)

- 3 Switches (2 for Bus, 1 for Star)

- 1 Router

- Copper Straight-Through Cables

Network Topology Diagram

Fig: Bus and Star Topology Connection

IP Addressing Scheme

| Device | Interface/PC | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC0 (Bus) | FastEthernet 0 | 192.127.7.2 | 255.255.255.0 | 192.127.7.1 |
| PC1 (Bus) | FastEthernet 0 | 192.127.7.3 | 255.255.255.0 | 192.127.7.1 |
| PC2 (Star) | FastEthernet 0 | 192.127.8.2 | 255.255.255.0 | 192.127.8.1 |
| PC3 (Star) | FastEthernet 0 | 192.127.8.3 | 255.255.255.0 | 192.127.8.1 |
| PC4 (Star) | FastEthernet 0 | 192.127.8.4 | 255.255.255.0 | 192.127.8.1 |
| Router Fa0/0 | Bus side | 192.127.7.1 | 255.255.255.0 | — |
| Router Fa1/0 | Star side | 192.127.8.1 | 255.255.255.0 | — |

Procedure

Step 1: Device Placement & Cabling

Connect PCs to switches, link switches to the router (Fa0/0 for Bus side, Fa0/1 for Star side).

Step 2: Assign IP Addresses to PCs

PC0> ipconfig 192.127.7.2 255.255.255.0 192.127.7.1

Similarly, For other PC’s with the help of above table.

- PC0 (Bus)

PC0> ipconfig 192.127.7.2 255.255.255.0 192.127.7.1

- PC1 (Bus)

PC1> ipconfig 192.127.7.3 255.255.255.0 192.127.7.1

- PC2 (Star)

PC2> ipconfig 192.127.8.2 255.255.255.0 192.127.8.1

- PC3 (Star)

PC3> ipconfig 192.127.8.3 255.255.255.0 192.127.8.1

- PC4 (Star)

PC4> ipconfig 192.127.8.4 255.255.255.0 192.127.8.1

Step 3: Configure Router Interfaces

Router> enable

Router# configure terminal

Router(config)# interface gigabitEthernet 0/0

Router(config-if)# ip address 192.127.7.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface gigabitEthernet 0/1

Router(config-if)# ip address 192.127.8.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# end

Router# write memory

Step 4: Test Connectivity

- From PC1 (Bus) → PC3 (Star):

- From PC4 (Star) → PC0 (Bus):

Observations

- Each topology communicated properly within its subnet.

- The router enabled communication between bus and star networks.

Result

The Bus and Star topologies were successfully interconnected, and cross-network pings verified communication.

Conclusion

This lab confirmed that different topologies can be interconnected using a router with separate subnets. Proper IP assignment and router configuration ensured seamless connectivity.